SRS Embedded Streaming Architecture

H.264/H.265 Live Streaming with SRT Protocol

Full-Duplex Interactive Voice, PTZ Control,

MQTT (EMQX) / WebSocket / DataChannel Signaling,

GOP Segmentation, VOD Aggregation & AWS S3 Storage

REVISED — Full-Duplex Voice + EMQX MQTT Integration

Comprehensive Technical Reference

# 1. Revised Overall System Architecture

This revision replaces the half-duplex push-to-talk (PTT) model with a full-duplex interactive voice architecture. The mobile player and embedded hardware maintain simultaneous bidirectional audio streams, enabling real-time conversation (like a phone call) rather than walkie-talkie style communication. Additionally, this revision introduces three signaling/control channel approaches: WebRTC DataChannel, MQTT via EMQX, and WebSocket, each with distinct trade-offs.

## 1.1 Revised High-Level Data Flow

┌─────────────────────────┐              ┌──────────────────────────────┐

│   EMBEDDED HARDWARE     │    SRT/UDP    │       SRS MEDIA SERVER       │

│   (Linux ARM/x86)       │──────────────>│       (v6.0 + Hooks)         │

│                         │               │                              │

│   H.264/H.265 + AAC    │               │  SRT ──> RTMP ──> WebRTC     │

│   (camera + ambient mic)│               │  (video + ambient audio OUT)  │

│                         │               │                              │

│   ┌───────────────────┐ │  WebRTC audio │  ┌──────────────────────┐   │

│   │ Opus Decoder +    │<│───────────────│──│ WebRTC rtc_to_rtmp   │   │

│   │ Speaker Playback  │ │  (voice IN)   │  │ (mobile voice IN)    │   │

│   └───────────────────┘ │               │  └──────────────────────┘   │

│                         │               │                              │

│   ┌───────────────────┐ │               │  WebRTC WHEP/WHIP endpoint   │

│   │ PTZ Motor Control │<│── MQTT/WS ────│──── Control API ────────────│

│   └───────────────────┘ │  (commands)   │                              │

└─────────────────────────┘               └───────────┬──────────────────┘

│

┌─────────────────────────────────────────────┤

│                    │                        │

v                    v                        v

┌──────────────────┐  ┌────────────────┐  ┌─────────────────────────┐

│  SEGMENT WORKER  │  │  EMQX MQTT     │  │   MOBILE PLAYER APP     │

│  (Python/Node)   │  │  BROKER         │  │   (iOS/Android)         │

│                  │  │                  │  │                         │

│  on_dvr webhook  │  │  PTZ commands   │  │   WebRTC WHEP playback  │

│  Validate GOP    │  │  Voice signaling │  │   FULL-DUPLEX voice     │

│  Concat segments │  │  Device status  │  │   (simultaneous talk +  │

│  Upload to S3    │  │  Presence       │  │    listen, like phone)  │

└──────────────────┘  └────────────────┘  │   PTZ via MQTT/WS/DC    │

└─────────────────────────┘

## 1.2 Key Architectural Change: Full-Duplex vs PTT

The fundamental difference is that the system now maintains two concurrent, independent audio streams:

| Aspect | Previous (PTT / Half-Duplex) | Revised (Full-Duplex Interactive) |
| --- | --- | --- |
| Audio direction | One direction at a time. User presses button to talk, releases to listen. | Both directions simultaneously. Continuous bidirectional audio like a phone call. |
| WebRTC tracks | Video track (recv) + Audio track (send, toggled on/off by PTT button). | Video track (recv) + Audio track (recv, ambient from camera) + Audio track (send, always active from mobile mic). |
| SRS config | rtmp_to_rtc=on only. rtc_to_rtmp optional. | rtmp_to_rtc=on AND rtc_to_rtmp=on both required. SRS bridges mobile audio back to RTMP for the device to consume. |
| Echo cancel | Not needed (only one direction active). | CRITICAL: Both the mobile app and embedded device must implement AEC (Acoustic Echo Cancellation) to prevent feedback loops. |
| Latency budget | Tolerant (500ms+ acceptable for walkie-talkie). | Strict: total round-trip must be under 400ms for natural conversation. Target 150ms one-way. |

## 1.3 Updated Project Folder Structure

The folder structure is updated to reflect full-duplex voice, EMQX MQTT integration, and the three signaling approaches:

srs-embedded-streaming/

├── embedded/

│   ├── encoder/

│   │   ├── srt_streamer.c            # SRT push (video + ambient audio)

│   │   ├── h264_pipeline.sh           # H.264 encode pipeline

│   │   ├── h265_pipeline.sh           # H.265 encode pipeline

│   │   ├── gop_controller.c           # Fixed-GOP keyframe enforcer

│   │   ├── ring_buffer.c/.h           # Zero-copy ring buffer

│   │   ├── backpressure_manager.c     # Adaptive bitrate

│   │   └── Makefile

│   ├── voice/                          # *** RENAMED from audio/ ***

│   │   ├── full_duplex_voice.c        # Full-duplex voice engine

│   │   ├── opus_codec.c               # Opus encode/decode (bidirectional)

│   │   ├── aec_processor.c            # Acoustic Echo Cancellation (Speex/WebRTC AEC)

│   │   ├── audio_mixer.c              # Mix ambient mic + incoming voice for speaker

│   │   ├── voice_websocket_client.c   # Approach B: WS voice transport

│   │   ├── voice_mqtt_client.c        # Approach C: MQTT voice signaling

│   │   └── alsa_io.c                  # ALSA capture/playback for mic+speaker

│   ├── ptz/

│   │   ├── ptz_daemon.c               # PTZ controller (ONVIF/Pelco-D/GPIO)

│   │   ├── ptz_protocol.h             # Abstract PTZ interface

│   │   ├── ptz_mqtt_listener.c        # Receive PTZ via MQTT subscription

│   │   ├── ptz_ws_listener.c          # Receive PTZ via WebSocket

│   │   ├── onvif_client.c

│   │   └── pelco_serial.c

│   ├── config/

│   │   ├── device.json

│   │   ├── srt_params.json

│   │   ├── buffer_policy.json

│   │   └── mqtt_config.json           # *** NEW: EMQX broker endpoint, topics ***

│   └── ...

│

├── srs-server/                         # (unchanged from Rev.1, see Section 3)

│   ├── conf/

│   │   ├── srs.conf                   # Updated: rtc_to_rtmp=on for voice return

│   │   └── ...

│   └── ...

│

├── emqx/                               # *** NEW: EMQX MQTT broker ***

│   ├── docker/

│   │   ├── Dockerfile                  # EMQX Enterprise or OSS image

│   │   └── docker-compose.emqx.yml     # EMQX cluster + persistence

│   ├── config/

│   │   ├── emqx.conf                   # Main EMQX config

│   │   ├── acl.conf                    # Topic ACL (device/user permissions)

│   │   └── auth_jwt.conf               # JWT authentication for mobile clients

│   ├── topics/

│   │   └── topic_schema.md             # MQTT topic namespace design

│   └── rules/

│       └── voice_routing_rule.json     # EMQX Rule Engine for voice signaling

│

├── segment-worker/                     # (unchanged from Rev.1, see Section 4)

│

├── mobile-player/

│   ├── ios/

│   │   ├── SRSPlayer/

│   │   │   ├── WebRTCClient.swift      # WHEP/WHIP client (video + voice)

│   │   │   ├── FullDuplexVoice.swift   # *** RENAMED: Always-on voice manager ***

│   │   │   ├── PTZController.swift      # PTZ via MQTT or WS or DataChannel

│   │   │   ├── MQTTClient.swift         # EMQX MQTT client (CocoaMQTT)

│   │   │   ├── WSClient.swift           # WebSocket client (Starscream)

│   │   │   ├── DataChannelManager.swift  # WebRTC DataChannel for signaling

│   │   │   ├── PlayerView.swift

│   │   │   └── SignalingClient.swift

│   │   └── Podfile

│   ├── android/

│   │   ├── app/src/main/java/.../

│   │   │   ├── WebRTCPlayer.kt

│   │   │   ├── FullDuplexVoiceService.kt  # *** Always-on voice ***

│   │   │   ├── PTZClient.kt

│   │   │   ├── EMQXMqttClient.kt          # Paho/HiveMQ MQTT client

│   │   │   ├── WSClient.kt                # OkHttp WebSocket

│   │   │   ├── DataChannelManager.kt

│   │   │   └── SignalingManager.kt

│   │   └── build.gradle

│   └── shared/

│       ├── signaling_protocol.md

│       ├── ptz_command_schema.json

│       └── voice_session_schema.json    # *** NEW: voice session lifecycle ***

│

├── control-api/

│   ├── src/

│   │   ├── server.js

│   │   ├── ptz_router.js

│   │   ├── voice_session_manager.js    # *** NEW: Full-duplex voice sessions ***

│   │   ├── mqtt_bridge.js              # *** NEW: EMQX integration ***

│   │   ├── device_registry.js

│   │   └── ws_handler.js

│   └── ...

│

└── infra/

├── terraform/

│   ├── srs_cluster.tf

│   ├── emqx_cluster.tf             # *** NEW: EMQX on EC2/ECS ***

│   ├── s3_vod.tf

│   └── cloudfront.tf

└── ...

# 2. Full-Duplex Interactive Voice Architecture

Full-duplex interactive voice means the mobile user and the person or environment at the embedded device location can hear each other simultaneously, with no button press required. This is architecturally more demanding than PTT because it requires simultaneous audio encoding/decoding, echo cancellation, and careful latency management on both endpoints.

## 2.1 Voice Path: Mobile to Device (Downlink Voice)

Mobile App Microphone

│

├──[WebRTC Audio Track, Opus encoded]──>

│

v

SRS Server (rtc_to_rtmp=on)

│  SRS transcodes Opus ──> AAC internally

│  and bridges to RTMP internal stream

│

├──[Option A: WebRTC DataChannel + audio]──> Device decodes directly

│

├──[Option B: RTMP/FLV pull by device]──> Device pulls FLV via HTTP

│

└──[Option C: MQTT audio signal routing]──> EMQX routes to device

│

v

Embedded Device

├── Opus Decoder (or AAC decoder)

├── AEC Processor (remove echo from speaker playback)

└── ALSA Speaker Output

## 2.2 Voice Path: Device to Mobile (Uplink Voice)

Embedded Device Microphone (ambient + local voice)

│

├── ALSA Capture ──> AAC Encoder

├── AEC Processor (remove echo from incoming mobile audio)

├── Muxed into MPEG-TS alongside H.264/H.265 video

│

└──[SRT/UDP]──> SRS Server

│

├── SRS demuxes audio from video

├── AAC ──> Opus transcode (SRS built-in)

│

└──[WebRTC Audio Track]──> Mobile App Speaker

Both paths operate simultaneously, creating a persistent bidirectional audio channel. The mobile user hears the camera's ambient audio (and any person speaking near the device) while simultaneously sending their own voice to the device's speaker.

## 2.3 Acoustic Echo Cancellation (Critical)

Without AEC, the voice loop creates feedback: the mobile user's voice plays on the device's speaker, gets picked up by the device's microphone, travels back through SRS, and plays on the mobile speaker, creating an echo or howling feedback loop. Both endpoints must implement AEC:

| Endpoint | AEC Implementation | Details |
| --- | --- | --- |
| Mobile App | Built-in WebRTC AEC. Google's libwebrtc includes AEC3 by default. Enable via RTCAudioSession config. | iOS: RTCAudioSession.useManualAudio = false (auto AEC). Android: WebRTC's AudioProcessing module handles AEC automatically. |
| Embedded Device | Software AEC using Speex DSP (speex_echo_cancellation) or WebRTC AudioProcessing Module (APM) compiled for ARM. | Feed the speaker output as the reference signal to the AEC. The AEC subtracts the expected echo from the mic capture before encoding to AAC/Opus. |

// embedded/voice/aec_processor.c - Speex AEC on embedded device

#include <speex/speex_echo.h>

#include <speex/speex_preprocess.h>

#define FRAME_SIZE 480     // 10ms at 48kHz

#define FILTER_LENGTH 4800 // 100ms tail length

typedef struct {

SpeexEchoState *echo_state;

SpeexPreprocessState *preprocess;

int16_t ref_buf[FRAME_SIZE];  // What we're playing to speaker

} aec_ctx_t;

void aec_init(aec_ctx_t *ctx) {

ctx->echo_state = speex_echo_state_init(FRAME_SIZE, FILTER_LENGTH);

int rate = 48000;

speex_echo_ctl(ctx->echo_state, SPEEX_ECHO_SET_SAMPLING_RATE, &rate);

ctx->preprocess = speex_preprocess_state_init(FRAME_SIZE, rate);

speex_preprocess_ctl(ctx->preprocess, SPEEX_PREPROCESS_SET_ECHO_STATE,

ctx->echo_state);

}

// Called for every audio frame captured from microphone

void aec_process(aec_ctx_t *ctx, int16_t *mic_in, int16_t *clean_out) {

// ref_buf contains what was just sent to the speaker

speex_echo_cancellation(ctx->echo_state, mic_in, ctx->ref_buf, clean_out);

speex_preprocess_run(ctx->preprocess, clean_out);

}

## 2.4 Full-Duplex Voice Engine on Embedded Device

// embedded/voice/full_duplex_voice.c

// Runs as a separate thread alongside the SRT video streamer

void* voice_engine_thread(void *arg) {

device_config_t *cfg = (device_config_t *)arg;

aec_ctx_t aec;

aec_init(&aec);

opus_encoder_t *enc = opus_encoder_create(48000, 1, OPUS_APPLICATION_VOIP);

opus_decoder_t *dec = opus_decoder_create(48000, 1);

alsa_handle_t *mic = alsa_open_capture("hw:0,0", 48000, 1);

alsa_handle_t *spk = alsa_open_playback("hw:0,0", 48000, 1);

// Connect to voice source (MQTT, WebSocket, or WebRTC DataChannel)

voice_channel_t *chan = voice_channel_connect(cfg);

while (running) {

// === DOWNLINK: Receive voice from mobile, play on speaker ===

uint8_t opus_pkt[512];

int recv_len = voice_channel_recv(chan, opus_pkt, sizeof(opus_pkt));

if (recv_len > 0) {

int16_t decoded[FRAME_SIZE];

opus_decode(dec, opus_pkt, recv_len, decoded, FRAME_SIZE, 0);

memcpy(aec.ref_buf, decoded, sizeof(decoded));  // AEC reference

alsa_write(spk, decoded, FRAME_SIZE);

}

// === UPLINK: Capture mic, AEC clean, encode, send ===

int16_t mic_raw[FRAME_SIZE], mic_clean[FRAME_SIZE];

alsa_read(mic, mic_raw, FRAME_SIZE);

aec_process(&aec, mic_raw, mic_clean);

uint8_t opus_out[256];

int enc_len = opus_encode(enc, mic_clean, FRAME_SIZE, opus_out, 256);

// Uplink audio is muxed into the SRT stream as AAC

// (or sent separately via voice channel for the independent voice path)

audio_mux_feed(cfg->srt_muxer, mic_clean, FRAME_SIZE);

}

}

# 3. Signaling & Control: Three Approaches Compared

The system needs a reliable channel for PTZ commands, voice session control, device status, and presence. Three approaches are available, each with different trade-offs. They can be used individually or combined (e.g., MQTT for PTZ + DataChannel for voice signaling).

## 3.1 Comparison Matrix

| Criterion | A: WebRTC DataChannel | B: WebSocket | C: MQTT (EMQX) |
| --- | --- | --- | --- |
| Transport | SCTP over DTLS over UDP. Part of the WebRTC peer connection. | TCP (ws://) or TLS (wss://). Persistent bidirectional connection. | TCP (mqtt://) or WebSocket (ws://mqtt). Pub/sub via EMQX broker. |
| Latency | Lowest (~50ms). Shares the same UDP path as WebRTC media. | Low (~80-150ms). TCP-based, subject to head-of-line blocking. | Low-Medium (~100-200ms). Broker adds one hop. QoS 0 is fastest. |
| Reliability | Configurable: ordered/unordered, reliable/unreliable per channel. | Reliable (TCP guarantees delivery). Reconnect logic needed. | Configurable QoS: 0 (fire-forget), 1 (at-least-once), 2 (exactly-once). |
| NAT traversal | Built-in via ICE/STUN/TURN. Works through most NATs. | Requires port exposure or reverse proxy. No NAT traversal built-in. | Client connects outbound to broker. Works through NAT naturally. |
| Scalability | P2P only. Each device needs its own DataChannel from SRS. | Server-managed. Scales with backend WebSocket infrastructure. | Best. EMQX handles 100M+ connections. Topic-based fan-out built-in. |
| Device side | Complex: requires full WebRTC stack on embedded device. | Moderate: lightweight WS library (libwebsockets or nopoll). | Simple: lightweight MQTT client (Paho C, NanoMQ, or Mosquitto lib). |
| Multi-viewer | Each viewer needs separate DataChannel. No native broadcast. | Server can broadcast to multiple WS connections. | Native pub/sub: one publish, all subscribers receive. |
| Voice audio | Voice carried natively via WebRTC audio tracks. Best for full-duplex. | Can carry Opus frames over WS binary messages. Higher overhead. | Not recommended for continuous audio (packet overhead too high). Use for signaling only. |
| PTZ commands | Good. Low latency. But requires active WebRTC session. | Good. Works independently of video session. | Best. Decoupled from video. Works even when no one is viewing. |
| Best for | When mobile app already has WebRTC connection and needs lowest latency. | Custom control protocol between app and a dedicated control server. | IoT-native device fleets, PTZ, presence, status. Fan-out to many viewers. |

## 3.2 Recommended Hybrid Approach

For a production system, the recommended architecture combines two approaches:

- Full-duplex voice: WebRTC audio tracks via SRS (Approach A). The WebRTC peer connection already exists for video playback. Adding bidirectional audio tracks to this connection is the most efficient path for full-duplex voice. SRS handles the SRT-to-WebRTC bridge for device-to-mobile audio, and the rtc_to_rtmp bridge for mobile-to-device audio.

- PTZ + device control + presence: MQTT via EMQX (Approach C). MQTT is the natural fit for IoT command-and-control. The embedded device subscribes to its PTZ command topic. Multiple mobile users can publish PTZ commands. EMQX's retained messages provide device status on connect. The decoupled pub/sub model means PTZ works independently of whether video is streaming.

- Fallback voice signaling: WebSocket (Approach B). If WebRTC fails (firewall blocks UDP), the app falls back to HTTP-FLV for video and WebSocket for bidirectional voice using Opus-over-WS binary frames.

# 4. MQTT Implementation with EMQX

## 4.1 EMQX Deployment

# docker-compose.emqx.yml

version: '3.8'

services:

emqx:

image: emqx/emqx-enterprise:latest

ports:

- '1883:1883'       # MQTT TCP

- '8083:8083'       # MQTT over WebSocket (for mobile browser fallback)

- '8084:8084'       # MQTT over WSS (TLS)

- '8883:8883'       # MQTT over TLS (mqtts://)

- '18083:18083'     # EMQX Dashboard

environment:

- EMQX_NAME=emqx@emqx.local

- EMQX_LOADED_PLUGINS=emqx_auth_jwt,emqx_rule_engine

volumes:

- ./emqx/config/emqx.conf:/opt/emqx/etc/emqx.conf

- ./emqx/config/acl.conf:/opt/emqx/etc/acl.conf

- emqx_data:/opt/emqx/data

restart: always

volumes:

emqx_data:

## 4.2 MQTT Topic Namespace Design

A well-structured topic namespace is critical for managing device fleets. The following schema separates concerns cleanly:

| Topic Pattern | Publisher | Subscriber | Purpose |
| --- | --- | --- | --- |
| devices/{device_id}/status | Device | API/App | Device online/offline, battery, temperature. Published as RETAINED message so new subscribers get latest state immediately. |
| devices/{device_id}/ptz/command | App/API | Device | PTZ commands: continuous_move, stop, goto_preset, set_preset. JSON payload with pan/tilt/zoom speeds. |
| devices/{device_id}/ptz/position | Device | App/API | Current PTZ position feedback. Published after each movement. Retained for latest position. |
| devices/{device_id}/voice/session | API | Device | Voice session lifecycle: {action: start/stop, caller_id, srs_stream}. Device uses this to know when to start/stop its voice engine. |
| devices/{device_id}/config/update | API | Device | Remote configuration: change bitrate, resolution, GOP, SRT params. Device applies and ACKs. |
| devices/{device_id}/stream/health | Device | API | Stream health metrics: SRT stats, buffer fill, dropped frames, bitrate. Published every 5 seconds. |
| fleet/broadcast | API | All Devices | Fleet-wide commands: firmware update notification, maintenance window, etc. |

## 4.3 MQTT Client on Embedded Device (Paho C)

// embedded/ptz/ptz_mqtt_listener.c

#include <MQTTClient.h>

#include <json-c/json.h>

#include "ptz_protocol.h"

#define MQTT_BROKER  "tcp://emqx-server:1883"

#define CLIENT_ID    "cam001"

#define PTZ_TOPIC    "devices/cam001/ptz/command"

#define STATUS_TOPIC "devices/cam001/status"

#define VOICE_TOPIC  "devices/cam001/voice/session"

// Last Will and Testament: mark device offline on disconnect

MQTTClient_willOptions will = {

.topicName = STATUS_TOPIC,

.message = "{\"online\":false,\"ts\":0}",

.retained = 1,

.qos = 1

};

int msg_arrived(void *ctx, char *topic, int len, MQTTClient_message *msg) {

json_object *obj = json_tokener_parse((char *)msg->payload);

if (strstr(topic, "ptz/command")) {

const char *cmd = json_object_get_string(

json_object_object_get(obj, "command"));

json_object *params = json_object_object_get(obj, "params");

if (strcmp(cmd, "continuous_move") == 0) {

float pan = json_object_get_double(

json_object_object_get(params, "pan_speed"));

float tilt = json_object_get_double(

json_object_object_get(params, "tilt_speed"));

ptz_continuous_move(pan, tilt, 0.0);

} else if (strcmp(cmd, "stop") == 0) {

ptz_stop();

} else if (strcmp(cmd, "goto_preset") == 0) {

int preset = json_object_get_int(

json_object_object_get(params, "preset_id"));

ptz_goto_preset(preset);

}

} else if (strstr(topic, "voice/session")) {

const char *action = json_object_get_string(

json_object_object_get(obj, "action"));

if (strcmp(action, "start") == 0) {

// Start full-duplex voice engine

voice_engine_start(json_object_get_string(

json_object_object_get(obj, "srs_stream")));

} else if (strcmp(action, "stop") == 0) {

voice_engine_stop();

}

}

json_object_put(obj);

MQTTClient_freeMessage(&msg);

MQTTClient_free(topic);

return 1;

}

void mqtt_connect_and_subscribe() {

MQTTClient client;

MQTTClient_create(&client, MQTT_BROKER, CLIENT_ID,

MQTTCLIENT_PERSISTENCE_NONE, NULL);

MQTTClient_setCallbacks(client, NULL, NULL, msg_arrived, NULL);

MQTTClient_connectOptions opts = MQTTClient_connectOptions_initializer;

opts.keepAliveInterval = 30;

opts.cleansession = 0;       // Persistent session for QoS 1

opts.will = &will;           // LWT for offline detection

opts.username = "cam001";

opts.password = "device_token_here";

MQTTClient_connect(client, &opts);

// Subscribe to PTZ commands and voice session control

MQTTClient_subscribe(client, PTZ_TOPIC, 1);    // QoS 1

MQTTClient_subscribe(client, VOICE_TOPIC, 1);

MQTTClient_subscribe(client, "devices/cam001/config/update", 1);

// Publish online status (retained)

char status[256];

snprintf(status, sizeof(status),

"{\"online\":true,\"ts\":%ld,\"fw\":\"2.1.0\"}",

time(NULL));

MQTTClient_publish(client, STATUS_TOPIC, strlen(status),

status, 1, 1, NULL);  // QoS 1, retained

}

## 4.4 MQTT Client on Mobile App (iOS Example)

// ios/SRSPlayer/MQTTClient.swift

import CocoaMQTT

class DeviceMQTTClient: CocoaMQTTDelegate {

let mqtt: CocoaMQTT

let deviceId: String

var onPTZPositionUpdate: ((Float, Float, Float) -> Void)?

var onDeviceStatusUpdate: ((Bool) -> Void)?

init(deviceId: String, broker: String, port: UInt16 = 8084) {

self.deviceId = deviceId

mqtt = CocoaMQTT(clientID: "mobile_\(UUID().uuidString.prefix(8))",

host: broker, port: port)

mqtt.enableSSL = true

mqtt.username = "mobile_user"

mqtt.password = "jwt_token_here"

mqtt.keepAlive = 30

mqtt.delegate = self

}

func connect() { mqtt.connect() }

func mqtt(_ mqtt: CocoaMQTT, didConnectAck ack: CocoaMQTTConnAck) {

// Subscribe to device status and PTZ position feedback

mqtt.subscribe("devices/\(deviceId)/status", qos: .qos1)

mqtt.subscribe("devices/\(deviceId)/ptz/position", qos: .qos0)

mqtt.subscribe("devices/\(deviceId)/stream/health", qos: .qos0)

}

// === Send PTZ Command ===

func sendPTZCommand(pan: Float, tilt: Float, zoom: Float = 0) {

let cmd: [String: Any] = [

"command": "continuous_move",

"params": ["pan_speed": pan, "tilt_speed": tilt,

"zoom_speed": zoom],

"ts": Date().timeIntervalSince1970

]

let data = try! JSONSerialization.data(withJSONObject: cmd)

mqtt.publish("devices/\(deviceId)/ptz/command",

withString: String(data: data, encoding: .utf8)!,

qos: .qos0)  // QoS 0 for PTZ (speed > reliability)

}

func stopPTZ() {

let cmd = "{\"command\":\"stop\"}"

mqtt.publish("devices/\(deviceId)/ptz/command",

withString: cmd, qos: .qos0)

}

// === Request Voice Session ===

func requestVoiceSession() {

let session: [String: Any] = [

"action": "start",

"caller_id": mqtt.clientID,

"srs_stream": "live/\(deviceId)"

]

let data = try! JSONSerialization.data(withJSONObject: session)

mqtt.publish("devices/\(deviceId)/voice/session",

withString: String(data: data, encoding: .utf8)!,

qos: .qos1)  // QoS 1 for session control

}

}

## 4.5 EMQX ACL Configuration

# emqx/config/acl.conf

# Devices can publish their own status/position/health

{allow, {username, {re, "^cam"}}, publish,

["devices/${username}/status",

"devices/${username}/ptz/position",

"devices/${username}/stream/health"]

}.

# Devices subscribe to their own command topics

{allow, {username, {re, "^cam"}}, subscribe,

["devices/${username}/ptz/command",

"devices/${username}/voice/session",

"devices/${username}/config/update",

"fleet/broadcast"]

}.

# Mobile users can publish PTZ commands, subscribe to status

{allow, {username, {re, "^mobile_"}}, publish,

["devices/+/ptz/command",

"devices/+/voice/session"]

}.

{allow, {username, {re, "^mobile_"}}, subscribe,

["devices/+/status",

"devices/+/ptz/position",

"devices/+/stream/health"]

}.

# Deny all other access by default

{deny, all}.

# 5. WebSocket Approach: Voice + Control

The WebSocket approach serves as both an alternative to MQTT and a fallback when WebRTC DataChannels are unavailable. It provides a persistent bidirectional TCP connection between the mobile app and the control API server, which relays commands to devices.

## 5.1 WebSocket Voice Transport (Fallback)

When WebRTC UDP is blocked by a firewall, voice audio can be carried over WebSocket binary frames. Each frame contains a single Opus packet (typically 20ms of audio at ~32kbps = ~80 bytes per frame). The overhead is small enough for real-time voice, though latency will be higher than WebRTC due to TCP head-of-line blocking.

// control-api/src/ws_handler.js - WebSocket voice relay

const WebSocket = require('ws');

const wss = new WebSocket.Server({ port: 3001, path: '/voice' });

// Map: device_id -> { deviceWs, mobileWs[] }

const voiceSessions = new Map();

wss.on('connection', (ws, req) => {

const params = new URL(req.url, 'http://localhost').searchParams;

const role = params.get('role');     // 'device' or 'mobile'

const deviceId = params.get('device_id');

if (!voiceSessions.has(deviceId)) {

voiceSessions.set(deviceId, { deviceWs: null, mobileWs: [] });

}

const session = voiceSessions.get(deviceId);

if (role === 'device') {

session.deviceWs = ws;

} else {

session.mobileWs.push(ws);

}

ws.on('message', (data, isBinary) => {

if (!isBinary) {

// JSON control message (PTZ, session control)

const msg = JSON.parse(data);

if (msg.type === 'ptz' && session.deviceWs) {

session.deviceWs.send(data);

}

return;

}

// Binary = Opus audio frame, relay to other side

if (role === 'device') {

// Device audio -> all connected mobile clients

session.mobileWs.forEach(m => {

if (m.readyState === WebSocket.OPEN) m.send(data);

});

} else {

// Mobile audio -> device

if (session.deviceWs?.readyState === WebSocket.OPEN) {

session.deviceWs.send(data);

}

}

});

ws.on('close', () => {

if (role === 'device') session.deviceWs = null;

else session.mobileWs = session.mobileWs.filter(m => m !== ws);

});

});

## 5.2 WebSocket Client on Embedded Device

// embedded/voice/voice_websocket_client.c (using libwebsockets)

#include <libwebsockets.h>

static int ws_callback(struct lws *wsi, enum lws_callback_reasons reason,

void *user, void *in, size_t len) {

voice_ws_ctx_t *ctx = (voice_ws_ctx_t *)user;

switch (reason) {

case LWS_CALLBACK_CLIENT_RECEIVE:

if (lws_frame_is_binary(wsi)) {

// Incoming Opus audio from mobile user

opus_decode(ctx->decoder, in, len, ctx->pcm_buf, FRAME_SIZE, 0);

memcpy(ctx->aec.ref_buf, ctx->pcm_buf, sizeof(ctx->pcm_buf));

alsa_write(ctx->speaker, ctx->pcm_buf, FRAME_SIZE);

} else {

// JSON control message (PTZ)

handle_ptz_command((char *)in, len);

}

break;

case LWS_CALLBACK_CLIENT_WRITEABLE:

// Send outgoing mic audio as binary Opus frames

if (ctx->opus_out_len > 0) {

unsigned char buf[LWS_PRE + 512];

memcpy(buf + LWS_PRE, ctx->opus_out, ctx->opus_out_len);

lws_write(wsi, buf + LWS_PRE, ctx->opus_out_len, LWS_WRITE_BINARY);

ctx->opus_out_len = 0;

}

lws_callback_on_writable(wsi);  // Request next write opportunity

break;

}

return 0;

}

# 6. WebRTC DataChannel Approach

If the embedded device has a full WebRTC stack (e.g., via libdatachannel or Google's libwebrtc compiled for ARM), DataChannels provide the lowest-latency signaling path because they share the same UDP connection as the media tracks. This avoids the need for a separate TCP connection to EMQX or a WebSocket server.

## 6.1 DataChannel for PTZ + Voice Signaling

In this approach, the SRS WebRTC peer connection between mobile app and SRS is extended with named DataChannels. However, note that SRS's built-in WebRTC does not natively support custom DataChannels. Therefore, DataChannel-based PTZ requires either: (a) a custom signaling server that establishes a separate WebRTC peer connection between the mobile app and the embedded device for DataChannel-only use, or (b) using SRS's WHIP endpoint to create a publish path from the mobile app that includes DataChannels, routed to the device via SRS forwarding.

The practical implementation typically uses approach (a), where the control API server acts as a WebRTC signaling server:

// mobile-player/ios/SRSPlayer/DataChannelManager.swift

import WebRTC

class DataChannelManager: NSObject, RTCDataChannelDelegate {

var ptzChannel: RTCDataChannel?

var voiceSignalChannel: RTCDataChannel?

var peerConnection: RTCPeerConnection?

/// Create DataChannels on the peer connection

func setupChannels() {

let ptzConfig = RTCDataChannelConfiguration()

ptzConfig.isOrdered = false     // PTZ: speed > ordering

ptzConfig.maxRetransmits = 0    // Fire-and-forget for continuous moves

ptzChannel = peerConnection?.dataChannel(

forLabel: "ptz", configuration: ptzConfig)

ptzChannel?.delegate = self

let voiceConfig = RTCDataChannelConfiguration()

voiceConfig.isOrdered = true    // Voice signals: maintain order

voiceConfig.isNegotiated = true

voiceConfig.channelId = 100

voiceSignalChannel = peerConnection?.dataChannel(

forLabel: "voice_signal", configuration: voiceConfig)

voiceSignalChannel?.delegate = self

}

func sendPTZ(pan: Float, tilt: Float, zoom: Float) {

let cmd = "{\"cmd\":\"move\",\"p\":\(pan)," +

"\"t\":\(tilt),\"z\":\(zoom)}"

let buffer = RTCDataBuffer(

data: cmd.data(using: .utf8)!, isBinary: false)

ptzChannel?.sendData(buffer)

}

func sendVoiceSessionStart() {

let msg = "{\"type\":\"voice\",\"action\":\"start\"}"

let buffer = RTCDataBuffer(

data: msg.data(using: .utf8)!, isBinary: false)

voiceSignalChannel?.sendData(buffer)

}

// Receive messages from device

func dataChannel(_ dc: RTCDataChannel, didReceiveMessageWith buffer: RTCDataBuffer) {

if dc.label == "ptz" {

// PTZ position feedback from device

let json = String(data: buffer.data, encoding: .utf8)!

handlePTZFeedback(json)

}

}

}

## 6.2 When to Use DataChannel vs MQTT vs WebSocket

| Use Case | DataChannel | MQTT (EMQX) | WebSocket |
| --- | --- | --- | --- |
| Full-duplex voice audio | BEST: Use WebRTC audio tracks (not DC itself for raw audio) | Signaling only (start/stop session) | FALLBACK: Opus-over-WS when WebRTC blocked |
| PTZ continuous move | Good (lowest latency, unreliable mode) | BEST (QoS 0, decoupled from video session) | Good (simple, but requires active connection) |
| PTZ goto preset | Good | BEST (QoS 1 for guaranteed delivery) | Good |
| Device presence/status | Not viable (requires active peer connection) | BEST (LWT + retained messages) | Possible (heartbeat-based) |
| Multi-viewer PTZ arbitration | Complex (need per-viewer channels) | BEST (shared subscription, central broker) | Moderate (server manages conflicts) |
| Fleet-wide broadcast | Not viable | BEST (single publish, all devices subscribed) | Possible (server iterates connections) |
| Works without video stream | No (requires WebRTC session) | Yes (fully independent) | Yes (independent TCP connection) |

# 7. Updated SRS Configuration for Full-Duplex Voice

The SRS configuration must be updated to enable bidirectional audio bridging. The key change is ensuring rtc_to_rtmp is enabled so that the mobile user's voice is bridged back to an RTMP stream that the embedded device can consume.

# Key changes in srs.conf for full-duplex voice

vhost __defaultVhost__ {

rtc {

enabled         on;

rtmp_to_rtc     on;       # Device->SRS->WebRTC (video + ambient audio)

rtc_to_rtmp     on;       # Mobile->SRS->RTMP (voice return to device)

pli_for_rtmp    6;

# CRITICAL for full-duplex: configure AAC-to-Opus bitrate

# Higher bitrate = better voice quality for conversation

aac_to_opus_bitrate  32000;  # 32kbps Opus for clear voice

}

play {

gop_cache       on;

gop_cache_max_frames 60;

queue_length    3;

mw_latency      50;        # Reduced for lower voice latency

}

publish {

mr              off;       # No merge-read for lowest latency

}

}

The full-duplex voice flow through SRS works as follows:

- Device publishes to SRS via SRT: Stream 'live/cam001' contains H.264/H.265 video + AAC ambient audio from the device's microphone.

- Mobile connects via WHEP: The mobile app pulls 'live/cam001' via WebRTC. SRS converts AAC to Opus and sends video + audio tracks to the mobile app. The mobile app simultaneously sends its own audio track (Opus) back to SRS.

- SRS bridges mobile audio to RTMP: With rtc_to_rtmp=on, SRS creates an internal RTMP stream (e.g., 'live/cam001_voice_return') containing the mobile user's Opus-to-AAC transcoded audio.

- Device consumes voice return: The embedded device pulls the voice return stream via HTTP-FLV (http://srs:8080/live/cam001_voice_return.flv) or subscribes via a lightweight RTMP client. It decodes the AAC audio and plays it through the speaker after AEC processing.

## 7.1 Device-Side Voice Return Consumer

# On the embedded device, pull the voice return stream

# This runs as a separate process alongside the SRT publisher

ffmpeg -f flv \

-i 'http://srs-server:8080/live/cam001_voice_return.flv' \

-vn \                            # Discard video (there is none)

-acodec pcm_s16le \              # Decode to raw PCM

-ar 48000 -ac 1 \                # 48kHz mono

-f s16le - | \

aec_and_play                      # Pipe to AEC processor + ALSA speaker

Alternatively, for lower latency, the device can use a lightweight librtmp or HTTP-FLV client in C to consume the voice return stream directly, bypassing FFmpeg's startup overhead.

# 8. Updated Docker Compose Stack

# docker-compose.yml - Complete stack with EMQX

version: '3.8'

services:

srs:

image: ossrs/srs:6

ports:

- '1935:1935'        # RTMP

- '8080:8080'        # HTTP (HLS/FLV/API)

- '1985:1985'        # HTTP API

- '8000:8000/udp'    # WebRTC

- '10080:10080/udp'  # SRT ingest

volumes:

- ./srs-server/conf/srs.conf:/usr/local/srs/conf/srs.conf

- ./data/dvr:/data/dvr

environment:

- CANDIDATE=${PUBLIC_IP}

restart: always

emqx:

image: emqx/emqx-enterprise:latest

ports:

- '1883:1883'        # MQTT TCP

- '8083:8083'        # MQTT over WebSocket

- '8084:8084'        # MQTT over WSS (TLS)

- '8883:8883'        # MQTTS (TLS)

- '18083:18083'      # EMQX Dashboard

volumes:

- ./emqx/config:/opt/emqx/etc/custom

- emqx_data:/opt/emqx/data

environment:

- EMQX_NAME=emqx@emqx.local

restart: always

hook-server:

build: ./srs-server/hooks

ports:

- '8085:8085'

volumes:

- ./data/dvr:/data/dvr

depends_on:

- redis

segment-worker:

build: ./segment-worker

volumes:

- ./data/dvr:/data/dvr

environment:

- AWS_ACCESS_KEY_ID=${AWS_KEY}

- AWS_SECRET_ACCESS_KEY=${AWS_SECRET}

- S3_BUCKET=my-vod-bucket

depends_on:

- redis

control-api:

build: ./control-api

ports:

- '3000:3000'        # REST + WebSocket

- '3001:3001'        # Voice WebSocket (fallback)

environment:

- EMQX_BROKER=mqtt://emqx:1883

- SRS_API=http://srs:1985

depends_on:

- emqx

- redis

redis:

image: redis:7-alpine

ports:

- '6379:6379'

volumes:

emqx_data:

## 8.1 Updated Port Reference

| Port | Protocol | Service | Purpose |
| --- | --- | --- | --- |
| 1935 | TCP | RTMP (SRS) | Internal RTMP bridge + voice return stream for devices |
| 8080 | TCP | HTTP (SRS) | HLS, HTTP-FLV (video + voice return), SRS web console |
| 1985 | TCP | HTTP API (SRS) | SRS management API |
| 8000 | UDP | WebRTC (SRS) | WHEP/WHIP: video playback + full-duplex voice |
| 10080 | UDP | SRT (SRS) | Ingest from embedded hardware devices |
| 1883 | TCP | MQTT (EMQX) | Device MQTT connections (PTZ, status, voice signaling) |
| 8083 | TCP | MQTT/WS (EMQX) | Mobile MQTT over WebSocket (browser/app fallback) |
| 8084 | TCP | MQTT/WSS (EMQX) | Mobile MQTT over secure WebSocket (production) |
| 18083 | TCP | Dashboard (EMQX) | EMQX management dashboard (admin only) |
| 3000 | TCP | Control API | REST API + WebSocket for PTZ/voice |
| 3001 | TCP | Voice WS | Opus-over-WebSocket voice fallback channel |

## 8.2 Decision Flowchart: Which Approach to Use

Mobile App Connection Decision:

1. Attempt WebRTC (WHEP) to SRS

├── SUCCESS ──> Video: WebRTC track

│                Voice: WebRTC audio tracks (full-duplex)

│                PTZ:   MQTT via EMQX (always)

│

└── FAIL (UDP blocked) ──>

2. Attempt HTTP-FLV from SRS

├── SUCCESS ──> Video: HTTP-FLV player

│                Voice: WebSocket Opus (full-duplex fallback)

│                PTZ:   MQTT via EMQX (always)

│

└── FAIL ──>

3. HLS from SRS (last resort)

├── Video: HLS player (5-10s latency)

├── Voice: WebSocket Opus (acceptable latency)

└── PTZ:   MQTT via EMQX (always)

Note: MQTT (EMQX) is ALWAYS used for PTZ regardless of video/voice path.

EMQX connects via MQTT-over-WebSocket (port 8084/WSS) from mobile,

which works through all firewalls since it's standard HTTPS.

End of Document — Revision 2 (Full-Duplex Voice + EMQX MQTT)